Deliverables

& Technical Requirements

DATE: 9/18/2025

Table 1: Sample Deliverable and Technical Requirements

| Deliverables/technical requirements table |  |  |  |
| --- | --- | --- | --- |
| # | Project deliverable/ functionality | Technical requirement | Test procedure |
| Structure |  |  |  |
| 1 | Aluminum Frame | Support full weight of robot + payload (≥10 lbs) with <5 mm deflection. | Load frame with max payload and measure deflection using calipers; verify within spec. |
| 2 | 3D Printed Mail Bucket | Two separate slots with max 15 lb capacity total; prevent parcel ejection during motion. | Load bucket with weighted parcels and drive over uneven surface; verify parcels remain secure. |
| 3 | Mail/Parcel Carrying Compartment | 2 separate carrying slots Max weight capacity 15 lbs | Place objects up until weight capacity in the compartment |
| 4 | Casters | Provide free rotation for smooth turning with <5 N drag force. | Measure force needed to rotate the caster using spring scale; verify <5 N. |
| Power |  |  |  |
| 5 | Battery + Buck Converters | Supply 12V motor power and 5V logic power for ≥2 hours of continuous operation. | Run the robot under full load until the battery voltage drops to 11V; record runtime and verify ≥2 hrs. |
| 6 | Emergency Stop An emergency stop mechanism will be integrated to provide immediate safety control. | Must cut motor power within 0.5 seconds of activation. | Engage E-stop while the robot is moving and measure time to stop using stopwatch or encoder feedback. |
| Sensing |  |  |  |
| 7 | IR Sensors (Obstacle Detection) | Detect obstacles ≥10 cm high within 0.5 m and trigger slow/stop command within 2 s. | Place objects at various distances; measure response time and verify correct stop/reroute behavior in ≥4/5 trials. |
| 8 | IMU (Inertial Measurement Unit) | Report robot orientation within ±5° accuracy during navigation. | Rotate robot known angles (90°, 180°) and compare IMU output to expected result; verify within tolerance. |
| Mobility |  |  |  |
| 9 | Autonomous Navigation | Make it from the starting point in the mail room to the two rooms | Run 5 consecutive trials on the defined route. For each room, measure stop position vs. target with a tape/marker, log total time from start to final stop, and record pass/fail per trial. |
| 10 | Obstacle Avoidance | Ensure when new items are in the path such as humans, boxes, chairs, etc to slow down and stop or go around. | Place human, box, and chair at randomized points along route; timestamp detection-to-action latency, verify stop/reroute and measure minimum clearance with a ruler/tape; repeat 5 trials. |
| 11 | Dual Motor Drivers + DC Motors | Robot must travel at 0.5–1.0 m/s with smooth acceleration and deceleration (no wheel slip). | Drive robot over 10 m track, measure time and verify average speed; visually confirm smooth motion. |
| 12 | Encoders | Provide positional feedback with ±2% error in distance traveled. | Drive known 5 m straight path and compare encoder reading to measured tape distance; verify error ≤2%. |
| Controllers & UI |  |  |  |
| 13 | Touchscreen Display UI | Must allow user to select delivery route and confirm start with <2 screen taps; UI must correctly transmit selection to the controller. | Perform 10 test inputs selecting various routes and confirm 100% match between selected and executed route. |
| 14 | High & Low Level Controllers | Must process route commands and send motor control signals within 50 ms latency between input and motor response. | Send control signal and measure response time with oscilloscope or software timestamp; verify <50 ms delay. |

Table 2: SMART Goals

| SMART Goals |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| # | Specific | Measurable | Attainable | Relevant | Timley |
| 1 | Support full robot weight with <5 mm deflection. | Caliper measures <5 mm deflection under load. | Aluminum frame strength is proven in robotics builds. | Ensures durability and safe transport. | Verified within Week 4 build phase. |
| 2 | Carry packages securely in two slots. | Packages remain in slots during 10/10 uneven tests. | 3D printing allows design adjustments. | Critical for safe package delivery. | Completed by prototype testing in Week 6. |
| 3 | Carry up to 15 lbs in compartment. | Load tested to 15 lbs without failure. | Readily achievable with compartment design. | Essential for package transport. | Verified by Week 6. |
| 4 | Casters rotate smoothly with <5 N force. | Spring scale test <5 N drag. | Standard casters meet requirement. | Enables smooth mobility. | Confirmed in assembly Week 5. |
| 5 | Provide ≥2 hrs power with battery + buck. | Runtime ≥2 hrs at full load. | Achievable with tested battery packs. | Ensures reliable operation. | Test by Week 7 integration. |
| 6 | Emergency stop cuts power <0.5s. | Stopwatch/encoder shows <0.5s. | Implemented with hardware switch. | Safety requirement. | Verified by Week 7 safety test. |
| 7 | Detect obstacles within 0.5 m. | Stop/reroute ≥4/5 trials. | IR sensors proven effective. | Prevents collisions. | Verified in Week 7 trials. |
| 8 | Report orientation within ±5°. | Compare IMU output to expected angle. | IMUs meet this tolerance. | Key for navigation accuracy. | Test during Week 6 navigation trials. |
| 9 | Navigate from mail room to rooms. | 5 consecutive successful trials. | Achievable with mapping + control. | Primary project function. | Fully tested by Week 8. |
| 10 | Avoid new path obstacles. | Stop/reroute correctly in 5 trials. | Sensors and code support behavior. | Ensures safe operation. | Validated by Week 8 testing. |
| 11 | Motors run 0.5–1.0 m/s smoothly. | Verified average speed on track. | Motor drivers support range. | Critical for movement. | Confirmed in Week 6 tests. |
| 12 | Provide feedback with ≤2% error. | Encoder vs actual path error ≤2%. | Off-the-shelf encoders meet this. | Ensures precise navigation. | Validated in Week 6–7 tests. |
| 13 | UI allows quick route selection. | Route selection correct in 10/10 trials. | Touchscreen programming feasible. | User-friendly operation. | Implemented by Week 7. |
| 14 | Process control commands <50 ms. | Measured latency <50 ms. | Hardware and software capable. | Needed for responsive control. | Verified in Week 7 integration. |

Deliverable 1

The robot’s frame will be constructed from aluminum to provide strength while keeping weight manageable.

Deliverable 2

The delivery bucket will be 3D printed and designed with two separate slots to allow for organized parcel handling.

Deliverable 3

This deliverable directly ties in with deliverable 2.

Deliverable 4

The robot will use casters to allow smooth maneuverability and free rotation, while minimizing resistance.

Deliverable 5

The robot will use a rechargeable battery system with buck converters to provide regulated voltage levels - 12V for the drive motors and 5V for logic circuits.

Deliverable 6

An emergency stop mechanism will be integrated to provide immediate safety control.

Deliverable 7

IR sensors will trigger a slow down or stop command based on height and range.

Deliverable 8

An IMU will be used to monitor and report the robot’s orientation during navigation. This is because of issues with encoders when there is floor slip and it causes the distance measured to be incorrect.

Deliverable 9

The robot must navigate autonomously from its starting point to the mail room to two designated rooms along a predefined route.

Deliverable 10

The robot will employ obstacle avoidance to safely navigate around dynamic and static objects such as humans, boxes, or chairs. It must slow, stop, or reroute appropriately when new obstacles are introduced into its path.

Deliverable 11

The mobility system will use dual motor drivers and DC motors to achieve controlled motion. The robot must maintain speeds while accelerating and decelerating smoothly without wheel slip.

Deliverable 12

Wheel encoders will provide positional feedback to enhance navigation accuracy.

Deliverable 13

The robot will feature a touchscreen display for route selection and system interaction. The interface must allow the user to select a delivery route and confirm the start in two screen taps.

Deliverable 14

The control system consists of high-level and low-level controllers, must process route commands and send motor control signals with minimal latency.