PortaMail

Product Requirement Specification

| ##### Document Number |  |
| --- | --- |
| ##### Revision | A |
| Release Date | 2/13/2026 |

| Approvals | Name | Date |
| --- | --- | --- |
| Team Leader | Sebastian Mejias | 2/13/2026 |
| Team Member | Henry Lovelace | 2/13/2026 |
| Team Member | David Ezuma | 2/13/2026 |
| Team Member | Pranay Dhoopar | 2/13/2026 |
| Team Member | Siddharth Ganti | 2/13/2026 |
| Customer/Sponsor | Dr. Logan Porter | 2/13/2026 |
| Advisor | Dr. Logan Porter | 2/13/2026 |

Revision History

|  | REV | DESCRIPTION | DATE |
| --- | --- | --- | --- |
|  | A |  | 2/13/2026 |

# 1.0 Scope

This document specifies the product requirements for the PortaMail Autonomous Indoor Mail Delivery System.The requirements defined establish the functional, performance, safety, environmental, and operational constraints necessary for design, implementation, verification, and validation of a single prototype unit.

The PortaMail system is an autonomous ground vehicle designed to transport mail and small packages within indoor hallway environments. The system shall autonomously navigate between predefined locations while avoiding static and dynamic obstacles and operating safely in shared human environments. The system is intended for prototype demonstration and academic evaluation.

# 2.0 Definitions

UGV – Unmanned Ground Vehicle

LiDAR – Light Detection and Ranging sensor

UI – User Interface

E-Stop – Emergency Stop mechanism

Payload – Combined transported package weight

Prototype – Single demonstration unit

# 3.0 Documents and Applicable Standards

Texas A&M University Capstone Design Requirements

IEEE 29148 – Systems and Software Requirements Engineering

ISO 12100 – Safety of Machinery (Reference)

ANSI/RIA R15.06 – Industrial Robot Safety (Reference)

# 4.0 Device Requirements

## 4.1 Functional Requirements

4.1.1 The product shall autonomously transport small mail and packages within indoor hallway environments.

4.1.2 The product shall support delivery of packages with a combined weight of up to 15 pounds.

4.1.3 The product shall allow users to select delivery destinations through a user interface.

4.1.4 The product shall navigate between a predefined home location and at least two delivery destinations.

4.1.5 The product shall detect and avoid static obstacles such as walls and furniture.

4.1.6 The product shall detect and avoid dynamic obstacles such as pedestrians.

4.1.7 The product shall return to the predefined home location upon completion of a delivery task.

4.1.8 The product shall provide confirmation of successful delivery via visual or audible indication.

4.1.9 The product shall include a manual emergency stop mechanism accessible to users.

4.1.10 The product shall securely retain packages during motion to prevent displacement.

## 4.2 Performance and Safety Requirements

4.2.1 The product shall operate continuously for a minimum of two hours on a single battery charge.

4.2.2 The product shall have a maximum operational speed of 1.5 meters per second.

4.2.3 The product shall reduce speed when obstacles are detected within one meter.

4.2.4 The product shall maintain a safe distance of at least 0.75 meters from pedestrians during operation.

4.2.5 The product shall automatically stop motion upon activation of the emergency stop.

4.2.6 The product shall enter a safe state in the event of critical sensor or software failure.

4.2.7 The product shall operate at a sound level not exceeding 65 dB during normal operation.

### 4.2.2 Human Factors

4.2.2.1 The device shall provide a touchscreen interface with large, readable buttons.

4.2.2.2 The device shall provide visual confirmation when a user selection is accepted.

4.2.2.3 The device shall automatically launch and display the user interface upon system power-up.

4.2.2.4 The device shall be fully operable via touch-based interaction.

## 4.2.3 Electrical Safety

4.2.3.1 The system shall include overcurrent protection for battery and motor circuits.

4.2.3.2 The system shall include proper insulation of exposed electrical wiring.

4.2.3.3 The battery system shall include overcharge and undervoltage protection.

## 4.2.4 System Reliability

4.2.4.1 The system shall fail to a safe state in the event of sensor malfunction.

4.2.4.2 The system shall automatically stop motion upon critical software failure.

## 4.2.5 Software Reliability and System Integrity

4.2.5.1 The system shall include a watchdog timer that resets the system in the event of software lockup.

4.2.5.2 The system shall boot to an operational state in under 60 seconds.

4.2.5.3 Navigation failure shall trigger a safe idle mode.

4.2.5.4 Delivery and error logs shall be stored in non-volatile memory.

4.2.5.5 Data shall be preserved during unexpected power loss.

4.2.5.6 Communication between subsystems shall include heartbeat monitoring signals.

4.2.5.7 Control loops shall operate at a minimum frequency of 50 Hz.

4.2.5.8 The software architecture shall use a modular ROS-based framework.

### 4.2.6 Environment

4.2.6.1	The system shall operate in indoor temperatures between 18°C and 27°C.

4.2.6.2  The system shall operate in relative humidity levels between 20% and 80% non-condensing.

4.2.6.4    The system shall withstand minor impacts equivalent to hallway collisions at ≤0.5 m/s without structural damage.

4.2.6.5    The system shall operate under standard indoor atmospheric pressure conditions (1 atm ± 10%).

4.3 Technical Performance Requirements

4.3.1 The navigation system shall maintain localization error within ±5 cm in indoor hallway environments.

4.3.2 The SLAM update rate shall be ≥10 Hz during motion.

4.3.3 The obstacle detection range shall cover 0.05 m to 5 m.

4.3.4 Navigation latency shall not exceed 200 ms from sensor input to motion command.

4.3.5 Closed-loop motor control shall maintain velocity error within ±5% of commanded speed.

4.3.6 Heading error shall remain within ±3° during straight-line motion.

4.3.7 Control loops shall operate at ≥50 Hz.

4.3.8 The system shall boot to operational state in under 60 seconds.

4.3.9 Battery charging time shall not exceed 3 hours.

4.3.10 The system shall operate between 11 V and 12.6 V supply range.

4.3.11 Voltage regulation shall maintain 5 V and 3.3 V rails within ±5% tolerance.

4.3.12 The system shall include low-battery auto-return or shutdown mode.

4.3.13 The system shall include watchdog timer reset functionality in the event of software lockup.

4.3.14 Navigation failure shall trigger safe idle mode.

# 4.4 Operational Constraints

4.4.1 The product shall be designed for indoor use only.

4.4.2 The product shall operate on level surfaces and standard hallway flooring.

4.4.3 The product shall be implemented as a single prototype unit.

4.4.4 The product shall require no permanent building modifications.

4.4.5 The product shall operate within the approved project budget ($1200).