Product Software Requirements

| ##### Document Number | 1 |
| --- | --- |
| ##### Revision | A |
| Release Date | 2/13/2026 |

| Approvals | Name | Date |
| --- | --- | --- |
| Team Leader | Sebastian Mejias | 2/12/2026 |
| Team Member | Henry Lovelace | 2/12/2026 |
| Team Member | Pranay Dhoopar | 2/12/2026 |
| Team Member | Siddharth Ganti | 2/12/2026 |
| Team Member | David Ezuma | 2/12/2026 |
| Customer/Sponsor | Dr. Logan Porter | 2/12/2026 |
| Advisor | Dr. Logan Porter | 2/12/2026 |

Revision History

|  | REV | DESCRIPTION | DATE |
| --- | --- | --- | --- |
|  | A | Pi, Teensy , and GUI software requirements | 2/13/2026 |

# 1.0 Scope

This document forms the Product Software Requirements for the PortaMail project.

PortaMail Software Requirements

1. Raspberry Pi 5 Interface

1.The Raspberry Pi 5 shall run Nav2 and SLAM Toolbox using LiDAR, IMU, and encoder sensor data.

2.The system shall use custom launch files to start all required nodes, including sensors, transforms, mapping, and navigation.

3.The Pi shall broadcast coordinate transforms linking the robot base frame to all sensors (LiDAR, IMU, and camera).

4.The system shall generate and maintain a 2D occupancy map for autonomous navigation.

5.The Pi shall fuse wheel encoder and IMU data to produce continuous odometry updates.

6.The odometry data shall be published to the ROS 2 network for navigation and localization.

7.The Xbox Series S controller shall connect via Bluetooth for teleoperation.

8.Joystick inputs shall be translated into velocity (cmd_vel) commands.

9.Velocity commands shall be verifiable through ROS 2 topic inspection.

10.A Micro-ROS Agent shall run on the Pi to bridge communication with the Teensy 4.0.

11.The Micro-ROS bridge shall support velocity command subscription and odometry publishing between systems.

2. Teensy 4.0 Interface

12.The Teensy 4.0 firmware shall operate as a Micro-ROS node written in C++.

13.The firmware shall subscribe to velocity commands from the Raspberry Pi.

14.The firmware shall publish odometry data back to the Raspberry Pi.

15.The firmware shall implement differential drive kinematics to convert linear and angular velocity into wheel-specific PWM signals.

16.The system shall enforce a maximum linear speed limit of 0.89 m/s.

17.The firmware shall include a deadman timeout that stops the motors if communication is lost.

18.The Teensy shall continuously read encoder tick data to track wheel rotation.

19.Encoder data shall be timestamped and synchronized with the Raspberry Pi through the Micro-ROS Agent.

3. LCD Touchscreen Interface

20.The LCD interface shall display two primary destination buttons labeled Room 1 and Room 2.

21.The interface shall display battery level status.

22.The system shall provide a Package Confirmation workflow allowing the user to confirm delivery.

23.The interface shall provide visual feedback when a button is pressed.

24.The interface shall display confirmation when a user action has been accepted.

25.The system shall transmit user selections (room choice, power command, confirmation) from the touchscreen to the Raspberry Pi.

26.The interface shall automatically start when the Raspberry Pi powers on.

27.The touchscreen interface shall be fully operable through touch input only.

28.The layout shall use large buttons and readable text for fast and simple interaction.