Product Testing Plan

| ##### Document Number |  |
| --- | --- |
| ##### Revision | A |
| Release Date | 2/20/2026 |

| Approvals | Name | Date |
| --- | --- | --- |
| Team Leader | Sebastian Mejias | 2/18/2026 |
| Team Member | David Ezuma | 2/18/2026 |
| Team Member | Henry Lovelace | 2/18/2026 |
| Team Member | Pranay Dhoopar | 2/18/2026 |
| Team Member | Siddharth Ganti | 2/18/2026 |
| Customer/Sponsor | Dr. Logan Porter | 2/18/2026 |

Revision History

|  | REV | DESCRIPTION | DATE |
| --- | --- | --- | --- |
|  | A | Original Release | 2/20/2026 |

# 1.0 Scope

This test will cover the validation of the PortaMail control system consisting of:

- Teensy 4.0 microcontroller

- Raspberry Pi 5

- Dual VNH5019 motor drivers

- 12V DC motors with quadrature encoders

- DFR1202 12V → 5V buck converter

- 12V battery input with protection circuitry

- RPLIDAR A2M12

- BNO055 IMU

- Raspberry Pi Camera

- 7” Touch Display

- Custom PCB interconnections

The purpose of testing is to verify:

- Electrical integrity and power stability

- Proper voltage regulation and distribution

- Correct motor driver functionality and direction control

- Encoder signal accuracy

- Sensor communication (I2C, UART, PWM)

- System stability under load

- Safe startup and shutdown behavior

Testing includes hardware validation, firmware verification, and subsystem integration testing.

| Test ID | Test Area | Component | Test Objective | Verification Method | Pass Criteria |
| --- | --- | --- | --- | --- | --- |
| TC-01 | Power Input | 12V Battery & Protection | Verify stable 12V input and protection behavior | Measure voltage at input test point | 12V within expected range, no overcurrent |
| TC-02 | Voltage Regulation | Buck Converter | Verify stable 5V output under no-load and load | Measure 5V rail with DMM/scope | 5V stable, no significant ripple |
| TC-03 | Microcontroller | Teensy 4.0 | Verify GPIO and PWM outputs | Oscilloscope measurement | 3.3V logic levels, stable PWM |
| TC-04 | Motor Control | VNH5019 Driver 1 | Verify forward and reverse operation | Command PWM and direction | Motor rotates correctly both directions |
| TC-05 | Motor Control | VNH5019 Driver 2 | Verify independent motor control | Command separate PWM | Both motors operate independently |
| TC-06 | Motor Feedback | Encoder 1 | Verify quadrature pulse detection | Rotate motor, monitor counts | Counts increase/decrease correctly |
| TC-07 | Motor Feedback | Encoder 2 | Verify stable signal at varying speeds | Run motor at multiple PWM levels | No missed pulses |
| TC-08 | Sensor Interface | BNO055 IMU | Verify I2C communication | Read orientation data | Valid, changing orientation values |
| TC-09 | Sensor Interface | RPLIDAR A2M12 | Verify 5V power and UART data | Read scan packets | Valid scan data stream |
| TC-10 | Integration | Teensy ↔ Motor Driver | Verify correct signal mapping | Functional motor test | No unintended activation |
| TC-11 | Integration | Teensy ↔ IMU | Verify stable communication during motor activity | Run motors + read IMU | No communication loss |
| TC-12 | Integration | Raspberry Pi ↔ Teensy | Verify serial communication | Send/receive test messages | Reliable bidirectional data |
| TC-13 | System | Full System Startup | Verify safe power-on behavior | Cold start test | No unexpected motor movement |
| TC-14 | System | Load Stability | Verify 5V rail under peak load | Run motors + sensors | No brownout or reset |
| TC-15 | Safety | Fault Condition | Verify safe behavior during reset | Reset MCU during operation | Motors stop safely |